Here’s a structured bullet-point summary of Reading 2.6: Introduction to Amazon VPC:

🔹 What Is a VPC?

- Amazon VPC (Virtual Private Cloud) = isolated network in AWS, similar to a traditional data center network.

- When creating a VPC, you define:

  - Name of the VPC.

  - Region (spans multiple Availability Zones).

  - IP range in CIDR notation (up to four /16 ranges).

- AWS provisions the network and IP addresses based on these settings.

🔹 Subnets

- Subnets = smaller networks inside a VPC (like VLANs in on-premises setups).

- Used for high availability and different connectivity options.

- To create a subnet, specify:

  - VPC association (e.g., 10.0.0.0/16).

  - Availability Zone (e.g., AZ1).

  - CIDR block (subset of VPC range, e.g., 10.0.0.0/24).

- EC2 instances are launched inside subnets.

🔹 High Availability

- Best practice: create at least two subnets in different AZs.

- Ensures redundancy and fault tolerance if one AZ fails.

🔹 Reserved IPs

- AWS reserves 5 IP addresses per subnet for routing, DNS, and network management.

- Example: VPC 10.0.0.0/22 → 1,024 IPs total → divided into four /24 subnets (256 IPs each).

- Usable IPs per subnet = 251 (after 5 reserved).

- Common design: VPC /16 with subnets /24 → large address space at both levels.

🔹 Gateways

- Internet Gateway (IGW):

  - Connects VPC to the internet.

  - Highly available and scalable.

  - Must be created and attached to the VPC.

- Virtual Private Gateway (VGW):

  - Connects VPC to another private network.

  - Acts as AWS-side anchor for VPN connection.

  - Requires a Customer Gateway (device/software on customer side).

  - Enables encrypted VPN connections between AWS and on-premises/private networks.